Bus Class –

Represents a bus (node) in a power system network.

Each bus has a unique index assigned automatically using a class-level counter.
A dictionary tracks all created buses by name.

Attributes:

- name: The name of the bus

- nominal_kv: The nominal voltage in kilovolts

Methods:

- get_bus_index(cls, name: str): Returns the index of the bus

Generator Class –

Represents a generator in a power system network.

A generator is connected to a single bus and controls voltage magnitude
while producing active power.

Attributes:

- name: The name / identifier of the generator

- bus1_name: The name of the where the generator is connected

- voltage_setpoint: Voltage magnitude setpoint in per-unit

- mw_setpoint: Active powet generation setpointi n megawatts (MW)

Load Class –

Represents a load in a power system network.

A load is connected to a single bus and consumes active power (MW)
and reactive power (MVAR).

Attributes:

- name: The name/identifier of the load

- bus1_name: Name of the bus where the load is connected

- mw: Active power consumption in megawatts (MW)

- mvar: Reactive power consumption in megavars (MVAR)

Transformer Class –

Represents a transformer in a power system network.

A transformer connects two buses and has series impedance (r + jx).

Attributes:

- name: The name/identifier of the transformer

- bus1_name: Name of the first bus (typically high voltage side)

- bus2_name: Name of the second bus (typically low voltage side)

- r: Resistance in per-unit or ohms

- x: Reactance in per-unit or ohms

Transmission Line Class –

Represents a transmission line in a power system network.

A transmission line connects two buses and has series impedance (r + jx)
and shunt admittance (g + jb).

Attributes:

- name: The name/identifier of the transmission line

- bus1_name: Name of the first bus

- bus2_name: Name of the second bus

- r: Series resistance in per-unit or ohms

- x: Series reactance in per-unit or ohms

- g: Shunt conductance in per-unit or siemens

- b: Shunt susceptance in per-unit or siemens

Circuit Class –

Represents a complete power system network.

The Circuit class serves as a container for all equipment objects
(buses, transformers, transmission lines, generators, and loads).

Attributes:

- name: The name of the circuit

- buses: Dictionary of buses in the circuit

- transformers: Dictionary of transformers in the circuit

- transmission_lines: Dictionary of transmission lines in the circuit

- generators: Dictionary of generators in the circuit

- loads: Dictionary of loads in the circuit

Methods:

- add_bus(self, name: str, nominal_kv: float): Adds bus to the circuit

- add_transformer(self, name: str, bus1_name: str, bus2_name: str, r: float, x: float): Adds transformer to the circuit

- add_transmission_line(self, name: str, bus1_name: str, bus2_name: str, r: float, x: float, g: float, b: float): Adds transmission line to the circuit

- add_generator(self, name: str, bus1_name: str, voltage_setpoint: float, mw_setpoint: float): Adds generator to the circuit

- add_load(self, name: str, bus1_name: str, mw: float, mvar: float): Adds load to the circuit

Classes Diagram –